| Hex Tiles (press molds) Art 140: Ceramics Mr. Kaplan | Tiles from the Mihrab of the Great Mosque of Isfahan Originally in Isfahan, Iran. Currently in the Metropolitan Museum of Art, NY |
| --- | --- |
| Tiles can be mass produced using a simple press mold. You will create an original tile in a pre-determined shape, but you must avoid undercuts or your mold will not work. |  |

|  | this side bevels in and won’t work; this side bevels out just a hair and will produce a successful mold. |
| --- | --- |
| Create an even slab that is 1/2 inch thick. Place a hexagon template on top and cut out the shape. You can also use a hexagon-shaped cutter. | Make sure you have at angle slightly greater than 91o angle between the table and the side of your hexagon. It must not bevel inward. |  |  |  |
| Place your blank hexagon on the tray inside the outline or in a single-tile pan. This is a good time to make sure your sides are still slightly greater than 90o. | Your top edges should not bow out either. It’s Okay if they are rounded, but they can’t be pinched outward. |  |  |  |
| You can work additively to create your design. Avoid undercuts like this. The flattened sphere goes inward and creates an unwanted undercut. | To eliminate the undercut, blend the bottom of the shape so it flows outward where it meets the tile. |  |  |  |
|  |  | ~OR~ |  |  |
| You can work subtractively, but still need to avoid undercuts. The channel on the left has a ridge blocking access to its lower parts. The top of the opening needs widened. The channel on the right is free of undercuts. | If it’s on a tray, build a clay reservoir around your tile. Ensure a finger’s width between the tile and the border, and that the reservoir is at least ½ an inch above the tile’s highest point. Your reservoir may share walls with the reservoirs of the other tiles on your tray. Add a thin coat of mold release like murphy’s oil soap. |  |  | If your tile is inside a pan or tray, add a coat of Vaseline or similar to any visible parts of the pan/tray including the walls and floor. |

Again, you will need to make sure your project is free from UNDERCUTS

| The cross-section of this form has undercuts | The cross-section of this form is free of undercuts |
| --- | --- |

| NAME__________________________________________________________ |  |  | Check One  |  |
| --- | --- | --- | --- | --- |
|  |  |  | Monday+Wednesday | Tuesday+Thursday |
| 03 Hex Tiles |  |  |  8:00-10:30 |  8:00-10:30 |
|  |  |  |  11:00- 1:30 |  11:00- 1:30 |
|  |  |  |  2:00- 4:30 |  |
|  |  |  |  5:00- 7:30 |  5:00- 7:30 |
| Craftsmanship | ON TARGET 5pts My mold is essentially functional. My positives are well finished with only slight marks left from the casting process | EXCELLENT 10pts My mold is highly functional. My positives are expertly finished and refined. Absolutely no marks are left from the casting process. |  |  |
| Radial Symmetry | ON TARGET 5pts Radial Symmetry is fairly consistent with only 1 or 2 minor deviations. | EXCELLENT 10pts Radial Symmetry is entirely consistent with no noticeable deviations. |  |  |
| Checklist | I completed at least three functional positives. |  |  |  5pt |
|  | I roughed up the back of 2 of my tiles to help them adhere to a surface for a mosaic. |  |  |  5pt |
|  | I smoothed the back of 1 of my tiles and removed any burrs and/or rough spots. |  |  |  5pt |
| Written Critique | Write a minimum of 1 well developed paragraph with five or more sentences. |  |  |  5pt |
|  | In your writing, describe your tiles to someone who couldn’t see or touch them |  |  |  5pt |
|  | In your writing, pick what shape would you make your tile if you weren’t limited to a hexagon |  |  |  5pt |
| Total |  |  | _____/50pts |  |